Henry’s law solubility constants Hcp = c/p (mmol L⁻¹ atm⁻¹).

computed for instance for the 34 °C values

For each gas, I pulled the recommended Hscp at 298.15 K and the temperature slope
B=dln⁡Hd(1/T)from Henry’s law database, then applied the standard relation:

ln⁡H(T2)  =  ln⁡H(T1)  +  B1T21T1

(This is the van ’t Hoff form used in Sander’s compilation; Hhere is the solubility variant Hscp≡c/p.)

Source values at 298.15 K (water):

- H₂: Hscp(298)=7.8×10-6 mol m-3 Pa-1, B=530 K.
Convert to M/atmwith 1 mol m-3 Pa-1=101.325 M/atm.

- CO₂: Hscp(298)=3.3–3.4×10-4 mol m-3 Pa-1, B≈2400 K.

- H₂S: Hscp(298)=1.0×10-3 mol m-3 Pa-1, B≈2100 K.

With T1=298.15 K and T2=307.15 K, Δ(1/T)=1307.15-1298.15=-9.820×10-5 K-1. Plugging in:

- H₂ @ 34 °C: scale factor exp⁡(B Δ)=exp⁡(530×-9.820 × 10-5)=0.949.
Hscp(298)=7.8 × 10-6 ⇒ 7.8× 10-6⋅101.325=7.90 × 10-4 M/atm.
Hscp(307)=0.949×7.90 × ⁣10-4=7.50× 10-4 M/atm
→ 0.750 mmol L⁻¹ atm⁻¹

- CO₂ @ 34 °C: factor =exp⁡(2400×-9.820 × 10-5)=0.790.
Hscp(298)≈3.3 × 10-4⇒0.0334 M/atm.
Hscp(307)=0.790×0.0334=0.0264 M/atm
→ 26.4 mmol L⁻¹ atm⁻¹.

- H₂S @ 34 °C: factor =exp⁡(2100×-9.820 × 10-5)=0.8135.
Hscp(298)=1.0 × 10-3⇒0.101325 M/atm.
Hscp(307)=0.8135×0.101325=0.0825 M/atm
→ 82.5 mmol L⁻¹ atm⁻¹