Summary

What MATLAB code calculates

Purpose
solving and fiting a two-phase (gas ↔ aqueous) kinetic model of the Mura 2024 high-pressure reactor experiment (≈60 bar, 36 °C) to reproduce gaseous H₂/CO₂ evolution and aqueous SO₄²⁻, formate, acetate, and calcium trends. Using Peng–Robinson EOS for gas non-ideality and Henry + Sechenov salting-out for liquid equilibria (ϕ–γ framework). then fitting 41 parameters (kinetics, transfers, inhibition, EOS binary interactions, carbonate initialization, and event pulses) by minimizing a weighted log residual against the experimental time series you read from Mura2024_Fig1_readoff_with_calcium.txt.

Experimental timeline captured in the model.

- Day 0: CH₄/CO₂ headspace at 60 bar (H₂=0), with CH₄ acting as cushion gas and CO₂ set from the first measured point.

- Day 7: make-up gas pulse with the same mix as the experiment (99:1 CH₄:CO₂); implemented as smooth pulses to avoid discrete jumps.

- Day 9: H₂ injection as a smooth pulse (amount fitted).

- Day 17: water addition (volume increase, affects phase inventories).

State variables (15):

nH₂(g), nCO₂(g), nCH₄(g), nH₂S(g), H₂(aq), CT (DIC), SO₄²⁻, formate, acetate, total sulfide \(S_t\), biomass X, Ca²⁺, FeS(s), Fe pool, Alk.

You track gas moles, aqueous concentrations, carbonate system (CT, Alk → pH, CO₂(aq), HCO₃⁻, CO₃²⁻), sulfide speciation, and iron sulfide precipitation.

Core calculations.

- Gas non-ideality: Peng–Robinson EOS → fugacity coefficients ϕᵢ for H₂/CO₂/CH₄ (with fitted binary kᵢⱼ).

- Liquid equilibrium: effective Henry constants Hcp (freshwater scales × Sechenov salting-out via ionic strength I); equilibrium Ceq for H₂, CO₂, H₂S.

- Mass transfer: kLa terms for H₂/CO₂/H₂S drive fluxes between phases.

- Carbonate system & pH: robust CT–Alk solver gives pH and speciation at each time step; initial CT₀ and Alk₀ are fitted with a soft pH(0) penalty around ≈6 ± 0.5.

- Microbial kinetics: four hydrogenotrophic routes: methanogenesis, sulfate reduction, formate production, and acetogenesis. Each uses Monod terms (H₂, CO₂, SO₄²⁻, formate), sulfide inhibition, product inhibition (formate/acetate), plus a logistic biomass cap with decay. Stoichiometry chains CO₂ sinks to avoid double counting. A pre-H₂ heterotrophic SR term (no H₂) operates only before day 9 to match early SO₄²⁻ fall.

- Mineral interactions: FeS precipitation limited by Fe pool and an HS⁻ supersaturation threshold; Ca²⁺ re-equilibration/dissolution toward an observed Ca_eq.

- Event pulses: smooth logistic windows for CO₂/CH₄ day 7 and H₂ day 9 feed rates (mmol day⁻¹), avoiding discontinuities.

Fitting & weighting.

- Objective: weighted log1p residuals for [H₂(g), CO₂(g), SO₄²⁻, formate, acetate, Ca²⁺] at experimental times. Early-time (\≤21 d) up-weighting for CO₂(g) & SO₄²⁻. Guards against negative gases and penalizes inconsistent pH(0). Optimized with lsqnonlin.

Diagnostics/outputs.

- Multi-panel species dashboard, pH(t), kinetic rates reporter, H₂S headspace diagnostic (ϕ–γ equilibrium vs. modeled), sulfur mass-balance (piecewise Vl), and an export file mura_2024_inc_rates.dat.

Data used (columns in your TXT):
time_d, H2_g_mmol, CO2_g_mmol, SO4_mM, formate_mM, acetate_mM, calcium_mM — read and used for fitting and plotting.

How this maps to the paper.
Our staged protocol (make-up day 7, H₂ injection day 9, water day 17), species observed, and analysis targets align with Mura et al., 2024 figures and method descriptions for gas/liquid trends at ~60 bar/36 °C in a low-salinity aquifer reactor.

Quick reference: your fitted parameter groups (41 total)

Kinetics & Monods (1–15): route rates (k_), yields (Y_), sulfide inhibitions (KI_), Monod half‑sat (K_).

Formate half‑sat (16): K_Form.

Biomass (17–18): decay b, logistic cap Xmax.

Transfer/Henry/Sechenov (19–27): kLa’s, Hcp scale factors, salting‑out ks.

Mineral & sulfide (28–30): Ca re‑equil., FeS precipitation, HS_sat.

PR EOS (31–33): binary interaction coefficients k_ij.

Carbonate init (34–35): CT₀_fit, Alk₀_fit.

H₂ pulse (36): day‑9 H₂ amount.

Product inhibition (37–38): KI_Ac_prod, KI_Form_prod.

CO₂ feed regulator (39): disabled when pulses active.

Day‑7 CO₂ make‑up (40): CH₄ forced to 99×CO₂.

Pre‑H₂ heterotrophic SR (41): r_sulf_org before day 9.